- Can you relate/connect Amdahl’s law with any of the design principles, mentioned below? Explain the reason with an example. [2]

|  | Design Principle |
| --- | --- |
| 1 | Performance via Prediction |
| 2 | Performance via Pipelining |
| 3 | Make the common case faster |
| 4 | Use Abstraction to Simplify Design |

## Define the term throughput and response time. How do these two differ, and why might one be prioritized over the other in different computing environments? [1+1+1+1]

- A computer runs a program in 20 seconds. A compiler optimization reduces instruction count by 15% and CPI by 10%, but the clock rate stays the same.

What is the new execution time? [3]

- To calculate the benchmark of a system, why do we take the geometric mean instead of only taking the average of the individual spec ratios? [3]

- A researcher is benchmarking the floating-point performance of an Intel Core i7 15th Gen processor using a subset of SPECfp benchmarks.
They record the reference execution times (from the official SPEC baseline machine) and the measured execution times (on the i7-15th Gen system). Calculate the final SPEC value, show all the necessary steps. [6]

| Benchmark | Reference Time (s) | Measured Time (s) |
| --- | --- | --- |
| 470.lbm | 2550 | 225 |
| 444.namd | 2950 | 210 |
| 453.povray | 3120 | 280 |
| 459.GemsFDTD | 4200 | 400 |
| 482.sphinx3 | 3750 | 350 |

- Mention if the following statement is true or false and also explain why.
A 4-core processor can always achieve a 4× speedup over a single-core version on any program. [1+1]

- Consider the following assembly code snippet for Intel Core-i7 15th Gen(x86).

| _start: mov rbx, [num] mov rax, rbx test rbx, 1 jnz .odd_case .even_case: imul rax, rbx .odd_case: imul rax, rbx imul rax, rbx .done: mov rdi, 0 mov rax, 60 syscall | 150ps Clock cycles |
| --- | --- |

  - Find the number of instructions in the above code. [1]

  - Find the average CPI. [2]

  - Find the clock cycle time. [1]

  - Find the time to execute this program. [2]

- A CPU has a MIPS rating of 6. A program consists of: 10 million instructions, of which 60% are arithmetic (CPI = 2), 30% are memory access (CPI = 3), and 10% are control (CPI = 1). Determine:

  - Average CPI [1]

  - Clock frequency of the processor [2]

  - Execution time of the program [2]

- You are optimizing the training pipeline for a new deep learning model. Profiling a single training epoch reveals the following execution time breakdown:
	# Data Loading & Preprocessing (CPU): 25% of the total time 
	# Tensor Matrix Math (GPU): 60% of the total time 
	#Graph Evaluation & I/O: 15% of the total time

Your project advisor wants to reduce the total training time by a factor of 3 (an overall speedup of 3.0x). The advisor suggests taking the entire budget and buying a massive, state-of-the-art GPU cluster to accelerate only the Tensor Matrix Math.

  - Mathematically prove whether the advisor's goal of a 3.0x overall speedup is possible or impossible under this plan. [2]

  - If you invest in a multi-core CPU upgrade that successfully makes the Data Loading & Preprocessing phase 5 times faster, exactly how much faster must you make the GPU Tensor Matrix Math to hit your advisor's strict 3.0x overall speedup target? [2]