- Given a machine code, what do we understand by looking at the Opcode field? [2]

- Suppose you have converted LD X9, 10(X21) to its equivalent RISC-V machine code. Now, based on which field, the machine will understand the size and extension type of the data being loaded into register X9? [3]

- Given the following 32-bit binary machine codes, determine the corresponding RISC-V assembly instruction. [2 + 2]
Use the provided opcode and funct reference table for identification

  - 0000000 01011 01010 000 01001 0110011

  - 0000000 00101 10001 101 10010 0010011

| Instruction | Opcode | Funct3 | Funct7 |
| --- | --- | --- | --- |
| add | 0110011 | 000 | 0000000 |
| xor | 0110011 | 100 | 0000000 |
| ld | 0000011 | 011 | - |
| srli | 0010011 | 101 | 0000000 |
| sub | 0110011 | 000 | 0100000 |

- Consider the following code:

int hishab(int i, int j) {

i = i + 8;

j = j - a[a [0]];

while (j < i) {

j = a [i];

i += 1;

}

return j;

}

k = 2;

l = 3;

a [0] = hishab(k, l);

  - Construct the equivalent RISC-V code of the above-mentioned C code. Assume that the variables k and l are in registers X5 and X6 respectively. a is a word array, which has its base address stored in X18. [15]

  - From your converted codes, pick any instruction from each of the three formats(R, I, S) and convert them into their corresponding binary values. [6]

Note: For opcode, funct3 or any other field values that you do not know just put “X”.