BRAC University

CSE340: Computer Architecture

Makeup Quiz

Duration : 20 Minutes

Total Marks : 20	Set: Abacus

ID:                                     Name: 						        Sec:

| 1. |  | Consider the following code: float a = fp_array [7]; float b = fp_array [6]; float c = fp_array [9]; int d = 78; float e = (float) d; if (a >= b) { e = a*b; } else { e = b*c; }; The base address of the fp_array array is in x9. The variables d and e are x19 and f19 respectively. The floating point variables are using double-precision.. |
| --- | --- | --- |
|  | a) | Construct the equivalent RISC-V assembly code for the given C program. |
|  | b) | Consider a = 2.789 and b = 3.892. Perform a*b using IEEE754 double precision floating point representation. Mention if there is any overflow or underflow. Your answer should show all the detailed steps and the final decimal value. Consider 5 digits after the floating point while performing the conversions. |
|  | c) | Explain the difference between the following instructions: - fcvt.s.w f3, x5 - fmv.x.w x3, f5 |
|  | d) | Consider the following numbers and different arbitrary-length IEEE 754 floating-point representations. Calculate if the numbers can be represented without any overflow/underflow. Your answer should show the detailed calculations. Number; Floating point number format. - 1.011111011* 232; Total length: 32 bits, fraction bits: 6. - 1.00*2-128; Total length: 32 bits, fraction bits: 18. - 0.00010110*244; Total length: 20 bits, fraction bits: 12. |