BRAC University

CSE340: Computer Architecture

Quiz#4

Duration : 20 Minutes

Total Marks : 20	Set: Abacus

ID:                                     Name: 						        Sec:

- Cache table:

| Index | Valid bit | Tag | Data(8 bits) |
| --- | --- | --- | --- |
| 000 | 1 | 100 | Data |
| 001 | 0 |  |  |
| 010 | 0 | 111 | Data |
| 011 | 1 | 001 | Data |
| 100 | 0 |  |  |
| 101 | 1 | 000 | Data |
| 110 | 1 | 010 | Data |
| 111 | 0 |  |  |

Memory accesses:

1. 36

2. 58

3. 22

4. 36

5. 62

  - For the above memory access, determine if it will be a hit or a miss. [5]

  - Calculate the hit and miss ratio. [4]

  - Why do we add layers to the memory hierarchy? What is the disadvantage of adding more layers? [4]

- Let's say we have 32 bit address space. we  have a cache where we can store                                                             1 byte data per block in cache (1 byte/block) .  We have 20 bit space for storing tag bits for each entry in cache. What is the total size of the cache ( considering tag, valid bit and data block) ?  [4]

| Index | Valid bit | Tag (20bit) | Data (1 byte) |
| --- | --- | --- | --- |

- What is the write through method? What problem arises in this method and how do we solve it? [3]

BRAC University

CSE340: Computer Architecture

Quiz#1

Duration : 20 Minutes

Total Marks : 20	Set: Byte

ID:                                     Name: 						        Sec:

- Cache table:

| Index | Valid bit | Tag | Data(8 bits) |
| --- | --- | --- | --- |
| 000 | 1 | 100 | Data |
| 001 | 0 |  |  |
| 010 | 1 | 111 | Data |
| 011 | 1 | 001 | Data |
| 100 | 0 |  |  |
| 101 | 1 | 000 | Data |
| 110 | 0 | 010 | Data |
| 111 | 0 |  |  |

Memory accesses:

1. 36

2. 58

3. 22

4. 36

5. 62

  - For the above memory access, determine if it will be a hit or a miss. [5]

  - Calculate the hit and miss ratio. [4]

  - Why do we add layers to the memory hierarchy? What is the disadvantage of adding more layers?. [4]

- Let's say we have 32 bit address space. we  have a cache where we can store                                                             1 byte data per block in cache (1 byte/block) .  We have 20 bit space for storing tag bits for each entry in cache. What is the total size of the cache ( considering tag, valid bit and data block) ?  [4]

| Index | Valid bit | Tag (20bit) | Data (1 byte) |
| --- | --- | --- | --- |

- What is the write back method? What problem arises in this method and how do we solve it? [3]