# Set Abacus:

- Solve

  - We need to find the average CPI for each segment separately.
Average CPI = (⅀(Instruction Counti * CPIi)) / Total Instruction Count

Average CPISegment A = (⅀(Instruction Counti * CPIi)) / Total Instruction Count
	         =  ( (400,000 * 50% * 2) + (400,000 * 30% * 4) + (400,00 * 20% * 3)
		 ) / 400,000
	         = (400,000 ( (50% * 2) + (30% * 4) + (20% * 3) )) / 400,000
	         = (50% * 2) + (30% * 4) + (20% * 3)
	         = (0.5 * 2) + ( 0.3 * 4) + (0.2 * 3)
	         = 1 + 1.2 + 0.6
	         = 2.8

Average CPISegment B = (⅀(Instruction Counti * CPIi)) / Total Instruction Count
	         =  ( (600,000 * 40% * 3) + (600,000 * 40% * 2) + (600,00 * 20% * 2)
		 ) / 600,000
	         = (600,000 ( (40% * 3) + (40% * 2) + (20% * 2) )) / 600,000
	         = (40% * 3) + (40% * 2) + (20% * 2)
	         = (0.4 * 3) + ( 0.4 * 2) + (0.2 * 2)
	         = 1.2 + 0.8 + 0.4
	         = 2.4

  - The program is divided into two segments. So the overall average CPI means CPI for the whole program(all segments).
Average CPI = (⅀(Instruction Counti * CPIi)) / Total Instruction Count

Average CPI = ( (Instruction CountSegment A * CPISegment A) +(Instruction CountSegment B     * CPISegment B) ) / Total Instruction Count

= ( (400,000 * 2.8) + (600,000 * 2.4) ) / (400,000 + 600,000) 
= (1,120,000 + 1,440,000) / (1,000,000)
= (2,560,000) / (1,000,000)
= 2.56

  - Clock cycle = Instruction Count * CPI
Total Clock cycle = Total Instruction Count * Average CPI
		    = 1,000,000 * 2.56
		    = 2,560,000



Alternate Solution:
Total Clock cycle = ⅀ ( Instruction Count * CPI ) 
		    = (400,000 * 50% * 2) + (400,000 * 30% * 4) + (400,00 * 20% *  3) + (600,000 * 40% * 3) + (600,000 * 40% * 2) + (600,00 * 20% * 2)
		   = 2,560,000

  - Execution Time = Instruction Count * CPI * Clock Rate
		  = (1,000,000) * 2.56 / 1.536 * 109
		  = 0.00167s
		  = 0.00167 * 103
		  = 1.67ms

  - Frequencyold = 1.536 GHz
After 20% increment, Frequencynew = 1.536 + (1.536 * 20%) GHz
				         = 1.8432GHz
As the CPI of memory instruction changes, the overall average CPI also changes.

CPI of memory instruction increased by 50%, so new CPI for memory instruction = 4 + 4 * 50% = 6

Average CPI = (⅀(Instruction Counti * CPIi)) / Total Instruction Count

Average CPInew = ( (Instruction CountSegment A * CPISegment A) +(Instruction CountSegment B     * CPISegment B) ) / Total Instruction Count
	         = ( (400,000 * 50% * 2) + (400,000 * 30% * 6) + (400,00 * 20% * 3)
		  + (600,000 * 40% * 3) + (600,000 * 40% * 2) + (600,00 * 20% * 2) ) ) / 1,000,000
	         = ( 400,000 + 720,000 + 240,000 + 720,000 + 480,000 + 240,000 ) / 1,000,000
	         = 2.8

Execution Timenew = ( Instruction Count * CPI ) / Clock Rate
		  =  ( 1,000,000 * 2.8 ) /  (1.8432 * 109)
		  = 0.00152s
		  = 0.00152 * 103 
		  = 1.52 ms
The new execution time is 18.75% ( (1.52 - 1.28) / 1.28 * 100%) more than the previous one.

  - MIPS = Clock Rate / (CPI * 106)
MIPSprevious = 600
MIPSnew = 700

MIPSnew = 700
=> ( Clock Ratenew / CPI * 106 ) = 700
=> Clock Ratenew = 700 * CPI * 106
=> Clock Ratenew = 700 * 2.56 * 106
=> Clock Ratenew = 1792000000
Clock Ratenew = 1.792 GHz

  - MIPS = Clock Rate / (CPI * 106)
MIPSprevious = 600
MIPSnew = 900

As the CPI of arithmetic instruction changes, the overall average CPI also changes.

CPI of arithmetic instruction increased by 50%, so new CPI for memory instruction = 2 + 2 * 20% = 2.4

Average CPI = (⅀(Instruction Counti * CPIi)) / Total Instruction Count

Average CPInew = ( (Instruction CountSegment A * CPISegment A) +(Instruction CountSegment B     * CPISegment B) ) / Total Instruction Count
	         = ( (400,000 * 50% * 2.4) + (400,000 * 30% * 4) + (400,00 * 20% * 3)
		  + (600,000 * 40% * 3) + (600,000 * 40% * 2) + (600,00 * 20% * 2) ) ) / 1,000,000
	         = ( 480,000 + 480,000 + 240,000 + 720,000 + 480,000 + 240,000 ) / 1,000,000
	         = 2.64

MIPSnew = 900
=> ( Clock Ratenew / CPInew * 106 ) = 900
=> Clock Ratenew = 900 * CPInew * 106
=> Clock Ratenew = 900 * 2.64 * 106
=> Clock Ratenew = 2376000000
Clock Ratenew = 2.376 GHz

Execution Timenew = Instruction Count * CPInew / Clock Ratenew
		      = ( 1,000,000 * 2.64 ) / (2.376 * 109) 
		      = 0.00111111111 s 
		      = 1.11 ms
Percentage change in execution time = (1.67 - 1.11 / 1.28) * 100%
					 = 33.53%

  - Execution Timeold = 1.67 ms (from d.)
Execution Timenew = 1.67 - (1.67 * 10%) = 1.67 - 0.167 = 1.503 ms
Only the floating point part of the execution time benefits from the new additional cores, so only the execution time for floating point operations change. The rest of the execution time is not affected.

Execution TimeFloating Point = ( IC * CPI ) / CR			   
			    = ( 600,000 * 20% * 2 ) / ( 1.536 * 109 )
			    = 0.15625ms
Execution TimeRest Instructions =  Execution TimeArithmetic + Execution TimeMemory + Execution TimeControl + Execution TimeLogical + Execution TimeI/O

= ( ICArithmetic * CPIArithmetic ) / CR + ( ICMemory * CPIMemory ) / CR + ( ICControl * CPIControl ) / CR + ( ICLogical * CPILogical ) / CR + ( ICI/O * CPII/O ) / CR

= ( 400,000 * 50% * 2 ) / ( 1.536 * 109 ) + ( 400,000 * 30% * 2 ) / ( 1.536 * 109 ) + ( 400,000 * 20% * 3 ) / ( 1.536 * 109 ) + ( 600,000 * 40% * 3 ) / ( 1.536 * 109 ) + ( 600,000 * 40% * 2 ) / ( 1.536 * 109 )
= ( 400,000 + 240,000 + 240,000 + 720,000 + 480,000 ) / ( 1.536 * 109 )
= ( 2,080,000 ) / ( 1.536 * 109 )
= 1.3542ms

As some part of the execution time is improved by adding additional cores, we can understand that this is an application of amdahl’s law. 

Tnew = Taffected / n + Tunaffected
=> 1.503 = 0.1562 / n + 1.3542
=> 0.1562 / n = 0.1488
n = 1.05

So, 1.05 times speedup is achieved.

# Set Byte

- Solve

  - We need to find the average CPI for each segment separately.
Average CPI = (⅀(Instruction Counti * CPIi)) / Total Instruction Count

Average CPISegment A = (⅀(Instruction Counti * CPIi)) / Total Instruction Count
	         =  ( (500,000 * 40% * 2) + (500,000 * 35% * 3) + (500,000 * 25% * 3)
		 ) / 500,000
	         = (500,000 ( (40% * 2) + (35% * 3) + (25% * 3) )) / 500,000
	         = (40% * 2) + (35% * 3) + (25% * 3)
	         = (0.4 * 2) + ( 0.35 * 3) + (0.25 * 3)
	         = 0.8 + 1.05 + 0.75
	         = 2.60

Average CPISegment B = (⅀(Instruction Counti * CPIi)) / Total Instruction Count
	         =  ( (500,000 * 30% * 4) + (500,000 * 50% * 4) + (500,000 * 20% * 3)
		 ) / 600,000
	         = (500,000 ( (30% * 4) + (50% * 4) + (20% * 3) )) / 500,000
	         = (30% * 4) + (50% * 4) + (20% * 3)
	         = (0.3 * 4) + ( 0.5 * 4) + (0.2 * 3)
	         = 1.2 + 2.0 + 0.6
	         = 3.8

  - The program is divided into two segments. So the overall average CPI means CPI for the whole program(all segments).
Average CPI = (⅀(Instruction Counti * CPIi)) / Total Instruction Count

Average CPI = ( (Instruction CountSegment A * CPISegment A) +(Instruction CountSegment B     * CPISegment B) ) / Total Instruction Count

= ( (500,000 * 2.60) + (500,000 * 3.8) ) / (500,000 + 500,000) 
= (1,300,000 + 1,900,000) / (1,000,000)
= (3,200,000) / (1,000,000)
= 3.20

  - Clock cycle = Instruction Count * CPI
Total Clock cycle = Total Instruction Count * Average CPI
		    = 1,000,000 * 3.20
		    = 3,200,000



Alternate Solution:
Total Clock cycle = ⅀ ( Instruction Count * CPI ) 
		    = (500,000 * 40% * 2) + (500,000 * 35% * 3) + (500,000 * 25% * 3) + (500,000 * 30% * 4) + (500,000 * 50% * 4) + (500,000 * 20% * 3)
		   = 3,200,000

  - Execution Time = Instruction Count * CPI / Clock Rate
		  = (1,000,000 * 3.20 ) / (2* 109)
		  = 0.0016s
		  = 0.0016 * 103
		  = 1.6 ms

  - Frequencyold = 2 GHz
After 20% increment, Frequencynew = 2 + (2 * 20%) GHz
				         = 2.4 GHz
As the CPI of memory instruction changes, the overall average CPI also changes.

CPI of memory instruction increased by 50%, so new CPI for memory instruction = 3 + 3 * 50% = 4.5

Average CPI = (⅀(Instruction Counti * CPIi)) / Total Instruction Count

Average CPInew = ( (Instruction CountSegment A * CPISegment A) +(Instruction CountSegment B     * CPISegment B) ) / Total Instruction Count
	         = ( (500,000 * 40% * 2) + (500,000 * 35% * 4.5) + (500,000 * 25% * 3) + (500,000 * 30% * 4) + (500,000 * 50% * 4) + (500,000 * 20% * 3) ) ) / 1,000,000
	         = ( 400,000 + 787,500 + 375000 + 600,000 + 1,000,000 + 300,000 ) / 1,000,000
	         = 3.4625

Execution Timenew = ( Instruction Count * CPI ) / Clock Rate
		  =  ( 1,000,000 *  3.4625 ) /  (2.4 * 109)
		  = 0.00144 s
		  = 0.00144 * 103 
		  = 1.44 ms
The new execution time is 10% ( (1.6 - 1.44) / 1.44 * 100%) less than the previous one.

  - MIPS = Clock Rate / (CPI * 106)
MIPSprevious = 625
MIPSnew = 750

MIPSnew = 760
=> ( Clock Ratenew / CPI * 106 ) = 750
=> Clock Ratenew = 750 * CPI * 106
=> Clock Ratenew = 750 * 3.20 * 106
=> Clock Ratenew = 2400000000
Clock Ratenew = 2.4 GHz

  - MIPS = Clock Rate / (CPI * 106)
MIPSprevious = 625
MIPSnew = 950

As the CPI of arithmetic instruction changes, the overall average CPI also changes.

CPI of arithmetic instruction increased by 50%, so new CPI for memory instruction = 2 + 2 * 20% = 2.4

Average CPI = (⅀(Instruction Counti * CPIi)) / Total Instruction Count

Average CPInew = ( (Instruction CountSegment A * CPISegment A) +(Instruction CountSegment B     * CPISegment B) ) / Total Instruction Count
	         = ( (500,000 * 40% * 2.4) + (500,000 * 35% * 3) + (500,000 * 25% * 3) + (500,000 * 30% * 4) + (500,000 * 50% * 4) + (500,000 * 20% * 3) ) ) / 1,000,000
	         = ( 480,000 + 525,000 + 375000 + 600,000 + 1,000,000 + 300,000 ) / 1,000,000
	         = 3.28

MIPSnew = 950
=> ( Clock Ratenew / CPInewI * 106 ) = 950
=> Clock Ratenew = 950 * CPInew * 106
=> Clock Ratenew = 900 * 3.28 * 106
=> Clock Ratenew = 2952000000
Clock Ratenew = 2.952 GHz

Execution Timenew = Instruction Count * CPInew / Clock Ratenew
		      = ( 1,000,000 * 3.28 ) / (2.952 * 109) 
		      = 0.00111 s 
		      = 1.11 ms
Percentage change in execution time = (1.6 - 1.11 / 1.6) * 100%
					 = 30.63%

  - Execution Timeold = 1.6 ms (from d.)
Execution Timenew = 1.6 - (1.6 * 5%) = 1.6 - 0.08 = 1.52 ms
Only the floating point part of the execution time benefits from the new additional cores, so only the execution time for floating point operations change. The rest of the execution time is not affected.

Execution TimeFloating Point = ( IC * CPI ) / CR			   
			    = ( 500,000 * 20% * 3 ) / ( 2 * 109 )
			    = 0.15 ms
Execution TimeRest Instructions =  Execution TimeArithmetic + Execution TimeMemory + Execution TimeControl + Execution TimeLogical + Execution TimeI/O

= ( ICArithmetic * CPIArithmetic ) / CR + ( ICMemory * CPIMemory ) / CR + ( ICControl * CPIControl ) / CR + ( ICLogical * CPILogical ) / CR + ( ICI/O * CPII/O ) / CR

= ( 500,000 * 40% * 2 ) / ( 2 * 109 ) + ( 500,000 * 35% * 3 ) / ( 2 * 109 ) + ( 500,000 * 25% * 3 ) / ( 2 * 109 ) + ( 500,000 * 30% * 4 ) / ( 2 * 109 ) + ( 500,000 * 50% * 4 ) / ( 2* 109 )
= ( 400,000 + 525,000 + 375,000 + 600,000 + 1,000,000 ) / ( 2 * 109 )
= ( 2,900,000 ) / ( 2 * 109 )
= 1.45 ms

As some part of the execution time is improved by adding additional cores, we can understand that this is an application of amdahl’s law. 

Tnew = Taffected / n + Tunaffected
=> 1.52 = 0.15 / n + 1.45
=> 0.15 / n = 0.07
n = 2.14

So, 2.14 times speedup is achieved.