# Set Clock Cycle

- 0xBACAD1E0 is a 32 bit value, addi can only add/load up to 12 bits, so we need lui instruction.
So for loading 0xBACAD1E0 into x22:
lui x22, BACAD
addi x22, x22, 1E0

N.B: Don’t be confused by the word “load”, it does not necessarily mean using the load instruction, load instructions are only used when we are transferring data from memory to register. Here we are asked to load into register x22, not from memory.

- Immediate value calculation:
The bne instruction jumps two instructions backward, so it jumps (2 * 4) = 8 locations in memory.
So the immediate value is 8
8 in 12 bit binary:
000000001000
After discarding the LSB and sign extending 1 bit we get:
000000000100

Now, as it is a backward jump, we need to convert it to its negative representation(2’s complement).

Reverting(1’s complement):
000000000100 -> 111111111011
Adding 1(2’s complement):
111111111011 + 1 = 111111111100

Final immediate value is: 111111111100

bne  x5, x0, loop
Now, converting to SB type machine code:

| 1 | 111111 | 00000 | 00101 | XXX | 1100 | 1 | XXXXXXX |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Imm [12] | Imm [10:5] | rs2 | rs1 | func3 | Imm [4:1] | Imm [11] | Opcode |

Machine code: 11111110000000101XXX11001XXXXXXX

- RISC-V Assembly:

addi x21, x0, 2 
addi x22, x0, 3
addi x23, x0, 4
addi x10, x21, 0
addi x11, x22, 0
addi x12, x23, 0
jal x1, kajKormo
sw x10, 48(x20)
lw x5, 48(x20)
slli x6, x5, 3
add x6, x5, x6
sw x6, 52(x20)
beq x0, x0, exit

kajKormo:
srl x5, x12, x11
or x10, x5, x12
jalr x0, 0(x1)

exit:

# Set Double Word

- 0xABACA1F0 is a 32 bit value, addi can only add/load up to 12 bits, so we need lui instruction.
So for loading 0xABACA1F0 into x22:
lui x22, ABACA
addi x22, x22, 1F0

N.B: Don’t be confused by the word “load”, it does not necessarily mean using the load instruction, load instructions are only used when we are transferring data from memory to register. Here we are asked to load into register x22, not from memory.

- Immediate value calculation:
The bne instruction jumps two instructions backward, so it jumps (2 * 4) = 8 locations in memory.
So the immediate value is 8
8 in 12 bit binary:
000000001000
After discarding the LSB and sign extending 1 bit we get:
000000000100

Now, as it is a backward jump, we need to convert it to its negative representation(2’s complement).

Reverting(1’s complement):
000000000100 -> 111111111011
Adding 1(2’s complement):
111111111011 + 1 = 111111111100

Final immediate value is: 111111111100

bne  x7, x0, loop
Now, converting to SB type machine code:

| 1 | 111111 | 00000 | 00111 | XXX | 1100 | 1 | XXXXXXX |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Imm [12] | Imm [10:5] | rs2 | rs1 | func3 | Imm [4:1] | Imm [11] | Opcode |

Machine code: 11111110000000111XXX11001XXXXXXX

- RISC-V Assembly:

addi x19, x0, 2 
addi x20, x0, 3
addi x21, x0, 4
addi x10, x19, 0
addi x11, x20, 0
addi x12, x21, 0
jal x1, kamKaj
sw x10, 48(x18)
lw x5, 48(x20)
slli x6, x5, 3
sub x6, x5, x6
sw x6, 52(x20)
beq x0, x0, exit

kamKaj:
srl x5, x12, x11
or x10, x5, x12
jalr x0, 0(x1)

exit: