BRAC University

CSE340: Computer Architecture

Quiz#3

Duration : 20 Minutes

Total Marks : 20	Set: Abacus

ID:                                     Name: 						        Sec:

- Draw a simplified datapath with control unit that can process only the following codes:

LD X20, 20(X22)

SD X22, 10(X20)   [12]

- Does the ALU control utilize instruction bits 30 and 14-12 to generate the output for the following instructions? Provide a justification for your answer.
LW X5, 0(X5)
SD X5,  0(X5)         [3]

- Consider the following instruction mix of a code snippet given below. For a single-cycle Datapath scenario, answer the following questions:

| R-type | I-type (non-ld) | Load | Store | Branch | Jump |
| --- | --- | --- | --- | --- | --- |
| 15% | 20% | 20% | 10% | 25% | 10% |

  - Determine the fraction of all instructions where register write signals are enabled [1.5]

  - Determine the fraction of all instructions where the final output come directly from the ALU [1.5]

  - Determine the fraction of all instructions where the processor needs to perform sign extension [2]

BRAC University

CSE340: Computer Architecture

Quiz#3

Duration : 20 Minutes

Total Marks : 20	Set: Byte

ID:                                     Name: 						        Sec:

- Draw a simplified datapath with control unit that can process only the following codes:

LD X25, 16(X6)

SD X6, 24(X25)   [12]

- Does the ALU control utilize instruction bits 30 and 14-12 to generate the output for the following instructions? Provide a justification for your answer.
LD X2, 0(X0)
SH X4,  0(X2)         [3]

- Consider the following instruction mix of a code snippet given below. For a single-cycle Datapath scenario, answer the following questions:

| R-type | I-type (non-ld) | Load | Store | Branch | Jump |
| --- | --- | --- | --- | --- | --- |
| 10% | 25% | 15% | 15% | 20% | 15% |

  - Determine the fraction of all instructions where register write signals are enabled [1.5]

  - Determine the fraction of all instructions where the final output come directly from the ALU [1.5]

  - Determine the fraction of all instructions where the processor needs to perform sign extension [2]