# Set Abacus

- The cache table will update after every memory access request.

| Index | Valid bit | Tag |
| --- | --- | --- |
| 000 | 1 | 100 |
| 001 | 0 |  |
| 010 | 1 | 111 111 |
| 011 | 1 | 001 |
| 100 | 1 | 100 |
| 101 | 1 | 000 |
| 110 | 1 | 010 111 |
| 111 | 0 |  |

| 100100 | miss |
| --- | --- |
| 111010 | miss ( as valid bit is zero) |
| 010110 | hit |
| 100100 | hit |
| 111110 | miss |

  - hit ratio = 25 =0.4
miss  ratio = 35=0.6

  - To balance speed, cost, and capacity—small, fast, expensive memories (like cache) are placed close to the CPU, while larger, slower, cheaper memories (like main memory and storage) are placed farther away, improving overall system performance.

Disadvantage of adding more layers:

It increases design complexity and can add extra access latency and management overhead (e.g., cache coherence and control logic).

- 212 (12 + 1 + 8) = 4452 bits

- Data is written to both cache and main memory on every write.
Problem:

High memory write traffic → slower performance.

Solution:

Use a write buffer to temporarily hold writes and reduce CPU stalls.

# Set Byte

- The cache table will update after every memory access request.

| Index | Valid bit | Tag |
| --- | --- | --- |
| 000 | 1 | 100 |
| 001 | 0 |  |
| 010 | 1 | 111 |
| 011 | 1 | 001 |
| 100 | 1 | 100 |
| 101 | 1 | 000 |
| 110 | 1 | 010 , 010 , 111 |
| 111 | 0 |  |

| 100100 | miss |
| --- | --- |
| 111010 | hit |
| 010110 | miss ( as valid bit 0) |
| 100100 | hit |
| 111110 | miss |

  - hit ratio = 25 =0.4

miss  ratio = 35=0.6

  - To balance speed, cost, and capacity—small, fast, expensive memories (like cache) are placed close to the CPU, while larger, slower, cheaper memories (like main memory and storage) are placed farther away, improving overall system performance.

Disadvantage of adding more layers:

It increases design complexity and can add extra access latency and management overhead (e.g., cache coherence and control logic).

- 212 (12 + 1 + 8) = 4452 bits

- Data is written only to cache; main memory is updated when the block is replaced.

Problem:

Main memory may have stale (inconsistent) data.

Solution:

Use a dirty (modified) bit to track changed blocks and update memory on eviction.