# CSE340: Computer Architecture

Quiz 2										Semester: Summer 26
Time: 40 minutes									Total Marks: 20
	
Name: ____________________________________  ID:___________________	Section: ________

Question 1 [10]

| Convert the corresponding code snippet to RISC-V assembly language. if (A[i*2] = = 0){ even = 4444; else{ odd = 1111; } Assume A is a half-word array, the base address of A is in X18. Variables i, even, odd in X20, X21, X22. Hard constraint: You can only use one shift operation in your code. | Answer: slli x5, x20, 2 add x5, x5, x18 lh x5, 0(x5) bne x5, x0, else lui x21, 1 addi x21, x21, 348 beq x0, x0, exit else: addi x22, x0, 1111 exit: |
| --- | --- |

Question 2 [6]

If there is a U-type instruction in the previous code of your question, convert it to its corresponding machine code. Assume the opcode for U-type instruction is 1010110. Your answer must be in hexadecimal.

Answer:

| Immediate | Rd | Opcode |
| --- | --- | --- |
| 0000 0000 0000 0000 0001 | 10101 | 1010110 |

Hex = 0x00001AD6

Question 3

Explain why we need U-type instructions. Then show how the U-type instruction changes the value of rd from the first question. [2+2]

Answer:

Addi can't add more than 12 bits of immediate.

x21 = 00….00 0000 0000 0001 0000 0000 0000

# CSE340: Computer Architecture

Quiz 2										Semester: Summer 26
Time: 40 minutes									Total Marks: 22
	
Name: ____________________________________  ID:___________________	Section: ________

Question 1 [10]

| Convert the corresponding code snippet to RISC-V assembly language. if (A[i*2] != 0){ odd = 5555; else{ even = 2222; } Assume A is a word array, the base address of A is in X18. Variables i, even, odd in X20, X21, X22. Hard constraint: You can only use one shift operation in your code and the destination register must be x5. | Answer: slli x5, x20, 3 add x5, x5, x18 lh x5, 0(x5) beq x5, x0, else lui x22, 1 addi x22, x22, 1459 beq x0, x0, exit else: addi x21, x0, 2222 exit: |
| --- | --- |

Question 2 [6]

If there is a modified I-type instruction in the previous code of your question, convert it to its corresponding machine code. Assume the opcode, funct3, funct6 for I-type instruction is 1010100, 001, 000100. Your answer must be in hexadecimal.

Answer:

| funct6 | immediate | rs1 | funct3 | rd | opcode |
| --- | --- | --- | --- | --- | --- |
| 0001 00 | 000011 | 10100 | 001 | 00101 | 1010100 |

Hex = 0x103A12D4

Question 3

- Explain why we need modified I-type instructions. [2]

Answer:

To ensure the maximum shifting is contained within 63.

- Show how the U-type instruction changes the value of rd from the first question. [2]

Answer:

x22 = 00….00 0000 0000 0000 0000 0001 0000 0000 0000

# CSE340: Computer Architecture

Quiz 2										Semester: Summer 26
Time: 40 minutes									Total Marks: 20
	
Name: ____________________________________  ID:___________________	Section: ________

Question 1 [10]

| Convert the corresponding code snippet to RISC-V assembly language. if (i = = 0){ A[i*2] = 4444; else{ i = i + 1; } Assume A is a half-word array, the base address of A is in X18. Variable i in X20. Hard constraint: You can only use one shift operation in your code. Assume rs1, rs2 of S type operation is x5, x6. | Answer: Bne x20, x0, else Lui x6, 1 Addi x6, x0, 348 Slli x5, x20, 2 Add x5, x5, x18 Sd x6, 0(x5) beq x0, x0, exit else: addi x20, x20, 1 exit: |
| --- | --- |

Question 2 [6]

Convert the S-type instruction in the if block in the previous code of your question to its corresponding machine code. Assume the opcode, funct3 for S-type instruction is 1000110, 101. Your answer must be in hexadecimal.

Answer:

| Immediate[5:11] | rs2 | rs1 | funct3 | immediate[0:4] | opcode |
| --- | --- | --- | --- | --- | --- |
| 0000000 | 00110 | 00101 | 101 | 00000 | 1000110 |

Hex: 0x0062D046

Note: in the question I wrote S type in else block however there is none in else block.

Writing either the if block's S type as machine code or that there is no S type instruction in else block will get full marks.

Question 3

- Explain why S-type instructions have the immediate field separated.

Answer:

So that it matches the format with other instructions for easier decoding.

- Show how the U-type instruction changes the value of rd from the first question. [2]

Answer:

x6 = 00…00 0000 0000 0000 0000 0001 0000 0000 0000

# CSE340: Computer Architecture

Quiz 2										Semester: Summer 26
Time: 40 minutes									Total Marks: 22
	
Name: ____________________________________  ID:___________________	Section: ________

Question 1 [10]

| Convert the corresponding code snippet to RISC-V assembly language. if (i ! = 0){ i = i + 1; else{ A[i*2] = 5555; } Assume A is a word array, the base address of A is in X18. Variable i in X20. Hard constraint: You can only use one shift operation in your code. Assume rd of U-type operation is x5. | Answer: beq x20, x0, else Addi x20, x20, 1 Beq x0, x0, exit else: lui x5, 1 addi x5, x5, 1459 Slli x6, x20, 3 Add x6, x6, x18 Sd x5, 0(x6) exit: |
| --- | --- |

Question 2 [6]

If there is a U-type instruction in the previous code of your question, convert it to its corresponding machine code. Assume the opcode for U-type instruction is 1010100. Your answer must be in hexadecimal.

Answer:

| Immediate | Rd | Opcode |
| --- | --- | --- |
| 0000 0000 0000 0000 0001 | 00101 | 1010100 |

Hex =  0x000012D4

Question 3

- Explain why we need funct3 in I-type instructions. [2]

Answer:

Sign and size of data for load instructions.

- Show how the U-type instruction changes the value of rd from the first question. [2]

Answer:

x5 = 00…00 0000 0000 0000 0000 0001 0000 0000 0000